| GPAA BLOCK C GROUND-RECEPTION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item no | Description | Unit | Qty | Rate | Amount |
|  | PRELIMINARIES |  |  |  |  |
|  | The agreement is to be the JBCC Series 2000 Principal Building Agreement prepared by the Joint Building Contract Committee July 2007 edition 5.0 code 2101 |  |  |  |  |
|  | PREAMBLES FOR TRADES The Model Preambles for Trades (2008 Edition) as published by the Association of South African Quantity Surveyors shall be deemed to be incorporated in these Bills of Quantities |  |  |  |  |
|  | GENERAL PRELIMINARIES To be excluded from the pricing. |  |  |  |  |
|  | The following is supplied by the Employer : Temporary services (water, electricity & sanitation), Contractors All Risk Insurance. To be included in the pricing : In this section the contractor is to price all project overheads including any responsibility directly or implied by the contract document, and including but not limited to the following: Site supervision, setting out, cleaning, jobbing, rubble removal, telephone and fax costs, transport costs, security, temporary stores, health & safety measures (including PPE, safety signage, safety files etc as per the construction regulations), workmen's compensation registration and upkeep, Public Liability Insurance of R 10m, printing of plans. |  |  |  |  |
|  | REFER TO ANNEXURE B FOR SPECIFIC PRELIMINARIES |  |  |  |  |
| 1 | Fixed Preliminaries Item | Item |  | R | R |
| 2 | Time Related Preliminaries Item | Item |  | R | R |
| 3 | Value Related Preliminaries Item | Item |  | R | R |

| GPAA BLOCK C GROUND-RECEPTION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item no | Description | Unit | Qty | Rate | Amount |
| BILL 1 | BILL NO. 1 |  |  |  |  |
|  | ALTERATIONS PREAMBLES For preambles see "Model Preambles to Trades" |  |  |  |  |
|  | SUPPLEMENTARY PREAMBLES View site before submitting his tender the contractor shall visit the site and satisfy himself as to the nature and extent of the work to be done and the value of the materials contained in the buildings or portions of the buildings to be demolished. No claim for any variations of the contract sum in respect of the nature and extent of the work or of inferior or damaged materials will be entertained |  |  |  |  |
|  | General |  |  |  |  |
|  | The contractor shall carry out the whole of the works with as little mess and noise as possible and with a minimum of disturbance to adjoining premises and their tenants. He shall provide proper protection and provide, erect and remove when directed, any temporary tarpaulins that may be necessary during the progress of the works, all to the satisfaction of the principal agent. Water supply pipes and other piping that may be encountered and found necessary to disconnect or cut, shall be effectually stopped off or grubbed up and removed, and any new connections that may be necessary shall be made with proper fittings, to the satisfaction of the principal agent Doors, fanlights, fittings, frames, linings, etc which are to be re-used shall be thoroughly overhauled before re-fixing including taking off, easing and re-hanging, cramping up, re-wedging as required and making good cramps, dowels, etc, and easing, oiling, adjusting and repairing ironmongery as necessary, replacing any glass damaged in removal or subsequently and stopping up all nail and screw holes with tinted plastic wood to match timber, unless otherwise described. Re-painting or re-varnishing is given separately Prices for taking out of doors, windows, etc shall include for removal of all beads, architraves, ironmongery, etc Pricing for items described as to be removed is to include for carting off-site and dumping at a dumpsite located by the contractor, dumping fees etc Prices for taking out and removing doors and frames shall include for removing door stops, cabin hooks, etc and making good floor and wall finishes to match existing With regard to building up of openings in existing walls, cement screeds and paving, granolithic, tops of walls, etc, shall be levelled and prepared for raising of brickwork Making good of finishes shall include making good of the brick and concrete surfaces onto which the new finishes are applied, where necessary The contractor will be required to take all dimensions affecting the existing buildings on the site and he will be held solely responsible for the accuracy of all such dimensions where used in the manufacture of new items (doors, windows, fittings, etc) Credit for materials being removed to be factored into the pricing |  |  |  |  |

| Item No |  |  | QUANTITY | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | SECTION 2 |  |  |  |  |
|  | BILL No. 01 |  |  |  |  |
|  | All the materials from alterations and demolishings or taken out will be the property of the contractor unless agreed otherwise |  |  |  |  |
|  | The contractor will be required to take all dimensions affecting the existing buildings on the site and he will be held solely responsible for the accuracy of all such dimensions where used in the manufacture of new items (doors, windows, fittings, etc) |  |  |  |  |
|  | With regard to building up of openings in existing walls, cement screeds and pavings, granolithic, tops of walls, etc, shall be levelled and prepared for raising of brickwork |  |  |  |  |
|  | Making good of finishes shall include making good of the brick and concrete surfaces onto which the new finishes are applied, where necessary |  |  |  |  |
|  | ALTERATIONS (PROVISIONAL) |  |  |  |  |
|  | PREAMBLES |  |  |  |  |
|  | For Preambles the tenderer is referred to the relevant clauses in a separate document Model Preambles for Trades (2008 Edition) |  |  |  |  |
|  | SUPPLEMENTARY PREAMBLES |  |  |  |  |
|  | View site |  |  |  |  |
|  | Before submitting his tender the contractor shall visit the site and satisfy himself as to the nature and extent of the work to be done and the value of the materials contained in the buildings or portions of the buildings to be demolished. No claim for any variations of the contract sum in respect of the nature and extent of the work or of inferior or damaged materials will be entertained |  |  |  |  |
|  | Explosives |  |  |  |  |
|  | No explosives whatsoever may be used for demolition purposes unless otherwise stated |  |  |  |  |
|  | General |  |  |  |  |
|  | The contractor shall carry out the whole of the works with as little mess and noise as possible and with a minimum of disturbance to adjoining premises and their tenants. He shall provide proper protection and provide, erect and remove when directed, any temporary tarpaulins that may be necessary during the progress of the works, all to the satisfaction of the principal agent |  |  |  |  |
|  | Doors, fanlights, fittings, frames, linings, etc which are to be re-used shall be thoroughly overhauled before refixing including taking off, easing and rehanging, cramping up, re-wedging as required and making good cramps, dowels, etc, and easing, oiling, adjusting and repairing ironmongery as necessary, replacing any glass damaged in removal or subsequently and stopping up all nail and screw holes with tinted plastic wood to match timber, unless otherwise described. Re-painting or re-varnishing is given separately |  |  |  |  |
|  | Water supply pipes and other piping that may be encountered and found necessary to disconnect or cut, shall be effectually stopped off or grubbed up and removed, and any new connections that may be necessary shall be made with proper fittings, to the satisfaction of the principal agent |  |  |  |  |
|  | Prices for taking out of doors, windows, etc shall include for removal of all beads, architraves, ironmongery, etc |  |  |  |  |
|  | Prices for taking out and removing doors and frames shall include for removing door stops, cabin hooks, etc and making good floor and wall finishes to match existing |  |  |  |  |
|  | REMOVAL OF EXISTING WORKS |  |  |  |  |
|  | Demolish existing partitioning and make good |  |  |  |  |
| 1 | Partitioning | m2 | 13 |  |  |
|  | Taking out and removing doors with locks (new doors and locks elsewhere measured) |  |  |  |  |
| 2 | Door size 813 x 2032mm high | No | 2 |  |  |
|  | Taking up and removing existing ceramic tiles and prepare floor to receive new tiles (measured elsewhere): |  |  |  |  |
| 3 | Ceramic tiles | m2 | 132 |  |  |
|  | Taking out and removing existing plaster and prepare to receive new (new plaster elsewhere measured) |  |  |  |  |
| 4 | Internal wall plaster | m2 | 327 |  |  |
|  | Repairing of existing concrete works |  |  |  |  |
| 5 | Repair and Repaint existing concrete canopy at entrance | Item |  |  |  |
|  | Repair of metalwork |  |  |  |  |
| 6 | Repair and paint existing balustrades. | Item |  |  |  |
|  | Mechanical Works |  |  |  |  |
| 7 | Existing aircon to be lowered to below new suspended ceiling | No | 1 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.1 |  |  |  |  |
|  | Alteration |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO.2 |  |  |  |  |
|  | BILL NO. 2 |  |  |  |  |
|  | CEILINGS, ETC. |  |  |  |  |
|  | SUPPLEMENTARY PREAMBLES |  |  |  |  |
|  | Descriptions: |  |  |  |  |
|  | Items described as "nailed" shall be deemed to be fixed with hardened steel nails or pins or shot pinned to brickwork or concrete |  |  |  |  |
|  | Items described as "plugged" shall be deemed to include screwing to fibre, plastic or metal plugs at not exceeding 600mm centres, and where described as "bolted" the bolts have been given elsewhere |  |  |  |  |
|  | PARTITIONS ETC |  |  |  |  |
|  | DEMOUNTABLE PARTITIONS |  |  |  |  |
|  | Glazing to be as follows: |  |  |  |  |
|  | GSA SmartGlass” ClearVue” Clear float glass, glazed in accordance with NBR N schedule 1 and SABS 0137. |  |  |  |  |
|  | Glass Properties : |  |  |  |  |
|  | ò Shading Coefficient = 0,71 to 1 |  |  |  |  |
|  | ò Light Transmission Level = 61 to 95 |  |  |  |  |
|  | GypWall Silent (48dB - 1 hour fire rating) Ultrasteel Stud Drywall consisting of stud and track system with 63,5mm Drywall Ultrasteel studs positioned at 600mm centres fixed into 63,5mm wide top and bottom track with Rhino Flam wire mesh surfaced fibre blanket fixed in position in cavity of partitioning clad on both sides with 12,5mm thick taper edged Rhino-Firestop board fixed with 25mm Drywall screws at 220mm centres and all corners fixed with 25 x 25mm corner beads and all joints to be taped and jointed as per manufacture's specifications. |  |  |  |  |
|  | All aluminium door frames, glazing sections, corners, transomes, cornices, skirtings, etc. shall be formed of natural anodised aluminium sections. Intersections and abutments are measured separately and descriptions shall be deemed to include any additional studs, corner beads, etc. |  |  |  |  |
| 1 | Partitioning 2.6m high with bottom tracks plugged and top tracks fixed to suspended ceiling tees | m2 | 12 |  |  |
|  | Doors |  |  |  |  |
| 2 | Extra over partition for 40mm semi-solid flush door 813 x 2030mm high with commercial veneer on both sides and concealed hardwood edge strips to vertical edges hung to and including standard natural anodised aluminium door frame with 305mm high fanlight filled in with and including clear float glass as above, with one pair of 100mm nylon washered aluminium hinges (lockset e/m) | No | 1 |  |  |
|  | SKIRTING, ETC: |  |  |  |  |
|  | Wrought meranti dado rail nailed: |  |  |  |  |
| 3 | Meranti dado rail stained mahogany nailed 850mm AFFL | m | 127 |  |  |
|  | SUSPENDED CEILINGS |  |  |  |  |
|  | 1200 x 600 x 9mm Ceiling tiles suspended by "Donn T38" or equal approved pre-painted exposed tee suspension grid system including hangers, necessary hold-down clips and wedges, etc: |  |  |  |  |
| 4 | Horizontal ceilings suspended not exceeding 1m below concrete soffits | m2 | 132 |  |  |
| 5 | Extra over ceiling for opening for 600 x 1200mm light fitting | No | 109 |  |  |
|  | Donn or equal approved cornices, perimeter trims, etc to suspended ceilings: |  |  |  |  |
| 6 | Type "SM25" pre-painted cornices, nailed | m | 127 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.2 |  |  |  |  |
|  | Ceiling, etc |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO. 2 |  |  |  |  |
|  | BILL NO. 3 |  |  |  |  |
|  | FLOOR COVERINGS, WALLS,ETC. |  |  |  |  |
|  | Provide and lay 4mm thick high density epoxy based heavy duty industrial screen flooring and topped with 1mm thick of self levelling compound to achieve total thickness of 5mm. |  |  |  |  |
| 1 | On floors | m2 | 30 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.3 |  |  |  |  |
|  | Floor Coverings, Walls, etc |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO 2 |  |  |  |  |
|  | BILL NO. 4 |  |  |  |  |
|  | METALWORK |  |  |  |  |
|  | SUPPLEMENTARY PREAMBLES |  |  |  |  |
|  | Descriptions |  |  |  |  |
|  | Descriptions of bolts shall be deemed to include nuts and washers |  |  |  |  |
|  | Descriptions of expansion anchors and bolts and chemical anchors and bolts shall be deemed to include nuts, washers and mortices in brickwork or concrete |  |  |  |  |
|  | Metalwork described as"holed for bolt(s)" shall be deemed to exclude the bolts unless otherwise described |  |  |  |  |
|  | Drawings |  |  |  |  |
|  | Tenderers are referred to architect's drawings annexed to this document for full details of the windows, doors, etc |  |  |  |  |
|  | ALUMINIUM WINDOWS, DOORS, ETC |  |  |  |  |
|  | AAAMSA guide |  |  |  |  |
|  | All windows, doors, etc shall comply with and meet the minimum recommended performance requirements as set out in the General Specification for Architectural Aluminium and Glass Products (Third Edition) as published by the Association of Architectural Aluminium Manufacturers of South Africa (AAAMSA) |  |  |  |  |
|  | Finish |  |  |  |  |
|  | The windows, doors, etc shall be natural anodised to a thickness of 25 micron and shall comply with SABS 999 and 1407 |  |  |  |  |
|  | Glass |  |  |  |  |
|  | Glazing to be with patent rubber gaskets with glazing beads and comply with BS 952. Thickness of glass shall be in accordance with table 1 (Part N : Glazing ). Safety glass shall comply with SABS 1263. The National Building Regulations shall be observed with regard to the specification of safety glass |  |  |  |  |
|  | Design indemnity |  |  |  |  |
|  | The contractor is to submit with his tender the "Form of Indemnity", annexed to this document, fully completed and signed |  |  |  |  |
|  | Drawings |  |  |  |  |
|  | Tenderers are referred to architect's drawings annexed to these bills of quantities for full details of windows, doors, etc |  |  |  |  |
|  | Pricing. |  |  |  |  |
|  | All window prices should include alluminium louvres as shown |  |  |  |  |
|  | General |  |  |  |  |
|  | Workshop drawings to be approved by the architect before manufacture |  |  |  |  |
|  | ALUMINIUM DOOR |  |  |  |  |
| 1 | Aluminium door size 813 x 2032mm | No | 2 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.4 |  |  |  |  |
|  | Metalwork |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO 2 |  |  |  |  |
|  | BILL NO. 5 |  |  |  |  |
|  | PLASTERING |  |  |  |  |
|  | For preambles see " Construction Works: Specification; General specification (PW371-A) Edition 2.0" |  |  |  |  |
|  | INTERNAL PLASTER |  |  |  |  |
|  | Cement plaster on brickwork finished with a steel float: |  |  |  |  |
|  | On internal walls | m2 | 327 |  |  |
|  | On narrow widths | m2 | 5 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.5 |  |  |  |  |
|  | Plastering |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO 2 |  |  |  |  |
|  | BILL NO. 6 |  |  |  |  |
|  | TILING |  |  |  |  |
|  | FLOOR TILING |  |  |  |  |
|  | 600 x 600mm Porcelain floor tiles (PC R250.00/m2 VAT excl. delivered to site) fixed with Ceresit Tylon adhesive to screeds (screeds elsewhere) with straight joints in both direction flush pointed with Ceresit Tylon 33 tinted jointing compound |  |  |  |  |
| 1 | On floor tiles | m2 | 132 |  |  |
| 2 | Skirting of 400 x 100mm high cut tile | m | 127 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.6 |  |  |  |  |
|  | Tiling |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO 2 |  |  |  |  |
|  | BILL NO. 7 |  |  |  |  |
|  | PAINTWORK |  |  |  |  |
|  | ON FLOATED PLASTER |  |  |  |  |
|  | Plascon cashmere to interior ivory white new cement plaster |  |  |  |  |
|  | Surface to be dry, sound and clean and cured for a minimum of 14 days, with a moisture content measured with a Doser Hygrometer (or equivalent), of BD 2 scale - 8% or less. Prime with one coat of Plascon Plaster Primer (UC 56) with an overcoating time of 16 hours and finish with two coats of Double Velvet (VEL) with 2 hours drying time between coats, for a maintenance cycle of 7 years in a C1 - inland environment. |  |  |  |  |
| 1 | Internal walls | m2 | 305 |  |  |
| 2 | External walls | m2 | 134 |  |  |
| 3 | Internal partition | m2 | 24 |  |  |
|  | ON WOOD |  |  |  |  |
|  | Three coats "Plascon" woodcare exterior waterbased varnish |  |  |  |  |
| 4 | Dado rail | m | 127 |  |  |
| 5 | Internal windows | Item |  |  |  |
|  | ROAD MARKING |  |  |  |  |
| 6 | Retro-reflective road marking paint 100mm wide broken or unbroken lines, white, yellow or red | m | 60 |  |  |
| 7 | Traffic island white or yellow markings | m2 | 8 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.7 |  |  |  |  |
|  | Paintwork |  |  |  |  |
| Item No |  |  | QUANTITY | RATE | AMOUNT |
|  | SECTION NO 2 |  |  |  |  |
|  | BILL NO. 8 |  |  |  |  |
|  | CAPENTERY |  |  |  |  |
|  | For preambles see " Construction Works: Specification; General specification (PW371-A) Edition 2.0" |  |  |  |  |
|  | TOPS, SHELVES, DOORS, MIRRORS, ETC |  |  |  |  |
|  | 20mm thick laminated glass |  |  |  |  |
| 1 | On top of Granite top by specialist | Item | 1 |  |  |
|  | Concrete Benches and Bins |  |  |  |  |
| 1 | Supply, delivery and installation of 2 (Two) 2500mm long concrete benches and 2 (Two) Bins by Specialists | Item | 1 |  |  |
|  | Steel palisade fence |  |  |  |  |
| 1 | Supply and erection of steel palisade fencing to match existing on site. Paint finish to match existing. | m | 6 |  |  |
|  | Blinds |  |  |  |  |
| 1 | Supply and install horizontal window blinds, 2000x2000mm installed to manufactures recommendation | Item | 1 |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Contingencies |  |  |  |  |
| 1 | Provide the sum of R35 000.00 (Thirty Five Thousand Rands) |  |  |  |  |
|  | for contingencies to be used at the discretion of the Project |  |  |  |  |
|  | Manager or deducted in whole or part if not required | Item |  |  |  |
|  | Carried to Summary |  |  |  |  |
|  | Section No.2 |  |  |  |  |
|  | Bill No.9 |  |  |  |  |
|  | Contigencies |  |  |  |  |
|  | SECTION SUMMARY |  |  |  |  |
|  |  |  | AMOUNT |  |  |
| 1 | ALTERATIONS |  |  |  |  |
| 2 | CEILINGS, ETC. |  |  |  |  |
| 3 | FLOOR COVERINGS |  |  |  |  |
| 4 | METALWORK |  |  |  |  |
| 5 | PLASTERING |  |  |  |  |
| 6 | TILING |  |  |  |  |
| 7 | GLAZING |  |  |  |  |
| 8 | CAPENTERY |  |  |  |  |
| 9 | CONTIGENCIES |  |  |  |  |
|  | CARRIED TO FINAL SUMMARY |  |  |  |  |
|  | FINAL SUMMARY |  | AMOUNT |  |  |
| 1 | PRELIMINARIES |  |  |  |  |
| 2 | BUILDINGS |  |  |  |  |
|  | Sub Total before VAT |  |  |  |  |
|  | Vat at the rate of 14% |  |  |  |  |
|  | Carried to Form of Tender |  |  |  |  |

ANNEXURE B – PRELIMINARIES:

PRELIMINARIES

The agreement is to be the JBCC Series 2000 Principal Building Agreement prepared by the Joint Building

Contract Committee July 2007 edition 5.0 code 2101

Note that Preliminaries are not referred to as a separate document but is contained within the above agreement

Tenderers are referred to the abovementioned document for the full intent and meaning of each clause thereof. These clauses are hereinafter referred to by clause number and heading only.

Where standard clauses or alternatives are not entirely applicable to this contract such modifications, corrections or supplements as will apply are given under each relevant clause heading and such modifications, corrections or supplements shall take precedence notwithstanding anything contrary contained in the abovementioned documents. Where any item is not relevant to this specific contract such item is marked N/A, signifying "not applicable".

PREAMBLES FOR TRADES

The Model Preambles for Trades (2008 Edition) as published by the Association of South African Quantity Surveyors shall be deemed to be incorporated in these Bills of Quantities and no claims arising from brevity of description of items fully described in the said Model Preambles will be entertained. Supplementary preambles are incorporated in these Bills of Quantities to satisfy the requirements of this project. Such supplementary preambles shall take precedence over the provisions of the said Model Preambles. The contractor's prices for all items throughout these Bills of Quantities must take account of and include for all of the obligations, requirements and specifications given in the said Model Preambles and in any supplementary preambles

SPECIFIC PRELIMINARIES

Where warranties for materials and/or workmanship are called for, the contractor shall obtain a written warranty, addressed to the employer, from the firm supplying the materials and/or doing the work and shall deliver same to the principal agent on the certified completion of the contract. The warranty shall state that workmanship, materials and installation are warranted for a specified period from the date of final completion and that any defects that may arise during the specified period shall be made good at the expense of the firm supplying the materials and/or doing the work, upon written notice to do so. The warranty will not be enforced if the work is damaged by defects in the construction of the building in which case the responsibility for replacement shall rest entirely with the contractor.

SHOP DRAWINGS

The contractor and/or nominated/selected sub-contractor shall prepare and submit, at his own expense, three (3) copies of shop drawings of all fabricated work, working or setting out drawings shop details and schedules to the principal agent for approval prior to commencement of manufacture. Correction of shop drawings shall not constitute a change in scope of work unless the contractor notifies the principal agent in writing within seven (7) days and shall not proceed with fabrication until so authorized by the architect.

SAMPLES

Samples are physical examples furnished by the contractor to illustrate materials, equipment or workmanship and to establish standards by which the work will be judged. The contractor shall furnish without delay such samples and "mock ups" as may be called for by the architect who may reject all material and workmanship not corresponding with the approved samples. Samples must be submitted in duplicate or as instructed and must allow a reasonable time for their consideration. Samples must be properly labelled with the project name and location, contractor's name, the type, finish composition of materials date of submission and the contract document reference in which the project is identified.

OVERTIME

Should overtime be required to be worked for any reason whatsoever, the costs of such overtime are to be borne by the contractor unless the principal agent has specifically authorized in writing, prior to the execution thereof, that costs for such overtime are to be borne by the employer.

CO-OPERATION OF CONTRACTOR FOR COST CONTROL

It is specifically agreed that the contractor accepts the obligation of assisting the professional consultants in implementing proper cost control in an effort to prevent the final building cost from exceeding the budget. It is specifically agreed that the professionals and the contractor will endeavour to agree and sign off variations within 14 days of them arising.

The contractor shall submit on a weekly basis an updated "Schedule of Information Required" to all parties concerned, indicating dates upon which such information and details are required in relation to the building programme. Timeous advanced notice is to be given by the contractor for information or drawings which are required on site. The contractor shall be responsible for the timeous notification to the Quantity Surveyor to enable him to call for tenders for all work to be executed by selected sub-contractors and shall, therefore, notify the quantity surveyor in writing at least fourteen (14) days prior to required availability date of these tender documents. No claim whatsoever will be afterwards admitted due to the contractor having failed to comply with this condition.

PROPRIETARY BRANDED PRODUCTS

The contractor shall take delivery of, handle, store, use apply and/or fix all proprietary branded products in strict accordance with the manufacturer's instruction after consultation with the manufacturer's authorized representative.

SITE INSTRUCTIONS

Site instructions issued on site are to be recorded in triplicate in a site instruction book which is to be maintained on site by the contractor.

LABOUR RECORD

At the end of each week the contractor shall provide the principal agent with a written record, in schedule form, reflecting the number and description of tradesmen and labourers employed by him and all

subcontractors on the works each day

HEALTH AND SAFETY ACT

Compliance with the current Occupation Health and Safety Act and all relevant and applicable regulations, especially the Construction Health & Safety Act, (as amended) for the duration of the contract.

HIV AND AIDS POLICY

Observation of all legislation, policies and by-laws regulating the "AIDS" endemic and "HIV" prevention, treatment and observation.

INSPECTION BY ARCHITECT / ENGINEER

The Architect / Engineer shall make such visits to the Works as he may from time to time deem necessary. In the event of any matter arising which the Contractor considers of such importance that the Architect / Engineer must be consulted, every reasonable attempt shall be made by the Contractor to communicate with him before proceeding with the point at issue.

It must, however, be borne in mind that the Architect is employed to ensure correct compliance with the terms of this contract, proper building procedures in accordance with the best traditions of the various trades and adequate finishes as specified and to his satisfaction. The Architect is thus in no way responsible for any act or omission on the part of the Contractor which may result in any patent or latent defects in materials or workmanship, breach or neglect of any local regulations. The Contractor therefore remains at all times responsible for any neglect, deviation or wrong act, whether the same be discovered before or after the final certificate, or any other certificate is issued.

DRAWINGS AND DIMENSIONS

Upon receipt of detail drawings for any works, the contractor shall, before putting that work in hand, ascertain that the dimensions given on the detail drawings correspond with the dimensions of any work already built. In the event of a discrepancy between the detail drawings and the work already built, the discrepancy shall be brought to the principal agent's attention who shall decide the procedure. All drawings, whatever their origin, are to be issued to the site, or to any other designated person or persons, only through the principal agent's office and shall bear the principal agent's office stamp and signature. An up to date register of all drawings issued to the contractor shall be kept on the works. Any other drawings used on the site will be used at the contractor's risk and should any work be incorrect due to the use of unauthorised drawings, the cost of rectifying such work shall be for the contractor's account. All drawings used on the works shall be properly mounted on suitable sheet material, or otherwise protected and kept in good condition. Any drawings becoming bleached or otherwise obscured so that they cannot be properly read shall be returned to the principal agent for replacement, as any errors due to misreading of damaged or obscured drawings shall be made good by the contractor at his own expense.

INJURY TO PERSONS OR PROPERTY

The Contractor shall in terms of the Compensation for Occupational Injuries and Diseases Act, 1993 (Act 130,

1993) (COID Act) be registered with the Director General of the Department of Labour as an Employer by the name in which the accepted tender was submitted and for the intended line of business pertaining to the Contract and the relevant COID Act Section 83 Assessments shall be kept fully paid up throughout the duration of the Contract. The Contractor must allow under this clause for all requirements as specified in the Compensation for Occupational Injuries and Diseases Act, 1993 (Act 130, 1993) and must fully adhere to all requirements and should allow to comply with all the relevant Legislation, as well as C.S.I.R Internal Services Health and Safety Specification - Annexure A. A safety audit will be done on a regular basis of all requirements

MEDIA RELEASES, ADVERTISING, ETC

All right of publication of articles in the media, together with any advertising relating to, or in any way connected with this project shall vest in the employer. The contractor, together with his sub-contractors shall not, without the written consent of the employer, cause any statement or advertisement to be printed, screened or aired by the media

PRIME COST AMOUNTS

Notwithstanding any information to the contrary in the contract document or preliminaries, references to "Prime Cost" or "PC" shall mean the cost at which an item or material is obtained inclusive of delivery to site and exclusive of value added tax. Unless otherwise stated installation is not included in the Prime Cost and the contractor is to include this in his rate. The prime Cost does not include any allowance for wastage and refers to cost per net measurable unit. The contractor is to allow for wastage, shrinkage, bulking etc.